LMP Professional

© 1989 R. Gonzalez

This document describes features of LMP Professional, a loudspeaker modelling program which predicts the response characteristics of multi-driver loudspeaker systems to aid in crossover design.  While LMP Professional does not provide automated optimization of crossover component values, it provides the user with the tools and performance needed to iteratively optimize the crossover himself, producing a loudspeaker which accurately reflects his own design priorities.

LMP Professional is not intended to perform vented and sealed enclosure analysis for optimizing low-frequency performance, though it may be used in conjunction with such software packages.

For a detailed discussion on loudspeaker modelling as it relates to LMP see "An Introduction to Frequency Response and LMP", in issues 1/87-3/87 of Speaker Builder Magazine, PO Box 494, Peterborough, NH 03458.  Backissues are available at a very reasonable cost.  (Have them ship UPS or you may have to wait a while!)

Other relevant writings in Speaker Builder are:

SB 2/87, p. 57:  corrections to main article

SB 3/87, p. 44-45:  response step (diffraction) theory, application

SB 3/87, p. 58:  impedance compensation, active crossovers with LMP

SB 1/88, p. 49-50:  phase shift questions

SB 2/88, p. 56-59:  modelling reverberant response with LMP

SB 3/88, p. 38:  minimum-phase crossover design with LMP

SB 3/88, p. 66-67:  step response and time delay

SB 4/88, p. 60:  LMP corner frequency questions

SB 5/88, p. 55-57:  more time delay comments

SB 2/89, p. 57-58:  LMP results for Weems project

SB 4/89, p. 61-65:  D'Appolito/Gonzalez on modelling time delay

SB 6/89, p. 64-65:  impedance compensation, notch filters, and LMP

____________________________________________________

____________________________________________________

MORE INFORMATION

In addition to the many excellent construction features in Speaker Builder , good references on loudspeaker design are Vance Dickason's "The Loudspeaker Design Cookbook" and Martin Colloms' "High Performance Loudspeakers", both available from Old Colony Sound Lab, PO Box 243, Peterborough, NH 03458.  Leo Beranek's "Acoustics" (McGraw-Hill, 1954) is a classic work available in some libraries (and now from Old Colony), and David Weems' "Building Speaker Enclosures" or "Great Sound Stereo Speaker Manual" are introductory texts, available at Radio Shack.

SPEAKER BUILDER INDEX

A Hypercard stack which indexes and crossreferences all articles and letters published in Speaker Builder Magazine to 4/90 was written by David Baldwin.  This allows you to conduct a keyword search — for example — for all articles/letters discussing the "compound driver" configuration, or to find all articles written by a particular author.  David Baldwin has ceased supporting and distributing it, but has authorized me to distribute it at $20/copy ppd. from my above address (we'll split the "profits").  I feel that this index makes the Speaker Builder backissues the single most useful reference on the art of speaker design available.

TOPBOX

Watch for the forthcoming Macintosh version of Joseph D'Appolito's TopBox program, for closed, vented, and bandpass box graphing and design, including equalization.  I've given it a good Macintosh interface (better than LMP Pro's I'll admit), and added the ability to make multiple graph overlays, transfer graphs and data to desktop publishing or spreadsheet programs, obtain high-resolution printer output, etc.

____________________________________________________

____________________________________________________

FILE MENU

The usual options are provided to "Open" and "Save" files containing information about the loudspeaker systems you are modelling.  Your LMP Professional disk comes with several such model files in the "Models" folder.

EDIT MENU

A special feature of the Edit menu is "Copy All".  This copies all the information about a particular driver to the Clipboard, so you can later "Paste" it into a different loudspeaker model.

MODEL MENU

You can set the number of drivers in your loudspeaker system using "Preferences".  To temporarily ignore a driver's contribution to the overall response, make it "inactive" here.

Choosing a driver from the Model menu brings up the "Model for Driver #" window.  Here you can set such parameters as sensitivity, polarity, depth displacement, etc. for your driver.  Some of these fields require more explanation.

LF/HF Corner Frequency:

This is the frequency where the driver's natural low or high frequency rolloff begins.  It is the corner frequency of the 2nd or 4th-order high-pass or low-pass filter which approximates the response of the driver.  For example, if a driver's damping ratio (Fig. 1) is D=0.7, then the corner frequency is that where the driver's response is -3 dB relative to its midband level.

LF/HF Damping Ratio (1/2Q):

This is a measure of the damping (freedom from peaking in the frequency response or "ringing" in the transient response) of the driver's low or high frequency rolloff.  It is defined as 1 over 2 times Q, where Q is the "resonance magnification" of the driver at its corner frequency.  For example, if the driver has a 6 dB peak at its LF corner frequency, then its Q is 2 (6=20*log10(2)), and its damping ratio is D=.25 (very poor!).  A driver whose response has dropped 6 dB (Q=.5) at its corner frequency is "critically damped" with a damping ratio of D=1/(2*.5)=1.

If the woofer's closed-box resonant frequency fcb and Qcb are known, the LF corner frequency equals fcb and the LF damping ratio equals 1/2Qcb.  (For vented systems, simply approximate the LF response.  The exact bass tuning usually has little effect on the crossover which is several octaves higher in frequency.)

For closed-back tweeters and midranges, you can estimate the LF corner frequency and damping ratio by comparing the manufacturer's frequency response curve to Figure 1, below.  First select the damping ratio curve whose shape resembles your driver's curve.  Then use the magnitude of this curve at the point where F/Fs=1 to find the corresponding corner frequency for your driver.  The same approach is used for the HF corner, except the figure is reversed.  If it is difficult to determine the damping ratio from the shape of the manufacturer's response curve, a good guess is the "maximally flat" alignment: D=0.7.

Figure 1. Damping Ratios (D) for LF corner (Fs)

Order of HF rolloff:

The above discussion applies only if the HF rolloff has roughly 12 dB/octave slope like a 2nd-order low-pass filter.  Some drivers drop more sharply above their HF corner frequency, approximating 24 dB/octave (4th-order filter).  In this case, the driver can be modelled using two cascaded (combined) 2nd-order low-pass filters.  Thus, a driver with a 6 dB peak at its HF corner frequency and a 4th-order rolloff is modelled with LMP as two cascaded 2nd-order filters with 3 dB peaks (Q=1.414), so its damping ratio should be entered in LMP as .35.

Interdriver Depth Displacement:

The depth displacement modelled with LMP refers to the physical difference in the distances between the listener and each driver.  It is not a measure of the so-called "acoustic center" of each driver, which actually varies with frequency.

Usually the best approach is to set the high frequency driver's depth displacement to zero, and measure the depth displacement of the remaining drivers relative to this driver.  For example, if the tweeter in a two-way system is flush with the front baffle and the woofer's cone is recessed, on average, 1.5" further from the listener, the woofer's depth displacement is -1.5.  If you instead set the tweeter's depth displacement to +1.5 and the woofer's to zero, you will obtain the same frequency-magnitude curve, but a different phase response due to the high frequency time delay (actually a time "lead").

Frequency and Height of Response Step:

A rise (usually about 6 dB) occurs in the frequency response of most drivers when they are mounted in an enclosure instead in a large baffle or wall.  This phenomena occurs because the driver's output is radiated in all directions at low frequencies but only in the forward direction at high frequencies.  For the purposes of LMP, the frequency of this response "step" is usually that where the wavelength is the same as the width of the enclosure face (F=13500/width).  You should reduce the value entered for the driver sensitivity by the height given for this step, since the manufacturer's sensitivity rating usually includes the effect of the response step.

It is often possible to equalize the response step using an appropriate crossover.

The response step is also termed "diffraction loss", taking the point of view that there is a 6 dB drop in response below a given frequency for drivers mounted in finite baffles, as sound begins to diffract around the baffle.  Indeed, diffraction can cause an irregular frequency response in the region of the response step unless care is taken to provide an assymetrical baffle layout or beveled baffle edges.

With the crossover set to "No Crossover" and the remaining drivers made inactive, you can produce a frequency response graph (see Response Menu, below) of the natural response of an individual driver to compare to the manufacturer's response curve.  Pay attention to the conditions under which this response curve was obtained:  was the driver in a large baffle or in a small enclosure?

Pressing the "New Crossover" button brings up a dialog box permitting the choice of 1st through 4th-order high, low, and band-pass crossovers.  First obtain a "cookbook" crossover to use as a starting point (for example, see Bullock's excellent 1985 SB series), and specify the component values in ohms, millihenries, and microfarads.  If the predicted response of your system using the "cookbook" crossover is not satisfactory, try experimenting with assymetrical crossover frequencies, driver polarity, depth displacement, sensitivity, etc.

Note that the driver itself is modelled in LMP as a resistor, which presumes that the driver's impedance curve is nearly flat within an octave or two of the crossover frequency.  This assumption is useful because crossover filters are often smoother and more predictable when operating into resistive loads.  In addition, a passive crossover designed in this way is easier to translate into an equivalent active version.  However, in many cases impedance compensation circuitry will be required to make the driver's impedance appear constant about the crossover frequency.  (See Knittel's article in SB 1/83, p. 11-14 or Bullock's in SB 1/85, p.19-22.)  Such circuitry should be placed between the crossover and the driver when you construct your system, to ensure the crossover will function as LMP predicted.  Impedance compensation circuitry is not shown in LMP's crossover illustrations.

Likewise, attenuating resistors (placed between the crossover and the driver or between the crossover and the impedance compensation) are not included in LMP's crossover alternatives, but can be modelled by changing the driver's sensitivity and impedance appropriately.  To model crossovers not provided in LMP, note you can specify 0 uF or 1000 mH to eliminate a capacitor or inductor (respectively), and 10000 uF/0 mH to short out a capacitor or inductor.  (Note that only the first three digits are visible in LMP's edit fields, so 10000 uF will appear on the screen as: 100 uF.)  To model an active crossover using LMP, simply calculate the component values for the equivalent passive crossover.

The technically-minded user can instead give the transfer function of the crossover, in factored or unfactored form.  This alternate method of crossover specification is very useful for rapid system development, and  is especially appropriate for designing active crossovers.  It is also possible to model aberrations in the driver's natural response by including them in the crossover transfer function definition.  Katsuhiko Ogata's "Modern Control Engineering" textbook (Prentice-Hall, 1970) offers a discussion of this form of analysis.

RESPONSE MENU

There are two forms of output produced by LMP Professional:  frequency response graph and waveform prediction.  First check the "Settings" dialog box, which specifies the upper and lower limits of the frequency response graph, the reference sensitivity, and the type of Macintosh being used.  This latter information is needed since the frequency response of the audio output jack on pre-Mac II machines requires a correction factor.  Even with this correction factor, the audio output of these machines is only accurate to about 7 kHz.  Mac II's are accurate to about 10 kHz.

The "Frequency Response" window displays the predicted frequency response (magnitude and phase) of your multi-driver loudspeaker system.  This corresponds to an anechoic loudspeaker measurement.

The "Play Waveform" feature is unique to LMP Professional.  Choose the type of waveform (square, sawtooth, or sine) and its fundamental frequency.  Press the "Play Model" button to see and hear the predicted response of the completed loudspeaker system to this test signal.  The "Play Ideal" button shows the response of a loudspeaker with perfect amplitude and phase characteristics.  Notice that the harmonics present in these test signals are multiples of the fundamental frequency.  Thus to investigate the effect of a crossover irregularity, choose a fundamental frequency less than or equal to the crossover frequency.  (The deviation from a perfect square or sawtooth wave in "Play Ideal"'s graphical output occurs because only the first 13 or 7 harmonics are being considered, respectively.  On the other hand, even greater deviation may be visible if you use an oscilloscope to observe the audio output of waveforms whose fundamental frequency is higher than about 500 Hz, due to the aforementioned 7-10 kHz high frequency limit of the Mac's DA converter.)

SUPPORT

Registered owners of LMP Professional may mail questions or comments with a self-addressed stamped envelope to: Ralph Gonzalez, PO Box 54, Newark, DE 19711.  I will do my best to answer questions but cannot guarantee a reply to requests for in-depth design assistance.  Please do not call me at home.

Good luck!